- Introduction:

The parts of the POS-Software which are obligate to verification have to be protected with checksums (here called value CS), created by an appropriate method (for instance CRC16). These checksums CS have also to be protected with checksums (here called value KW), created by the CRC16-method in use with a polynome P, which is known only by the manufacturers of the scale and the POS-system. CS and KW both have a length of 16 bit.

Remark: KW represents the rest of the polynome-division CS / P.

The checkout-dialogue 06 presents for the scale the possibility, to request 1-5 pairs of values CS/KW in cyclic intervals from the POS-system and check their validity. In case of invalidity no weight-results will be given from the scale to the POS-system, i.e. dealing with the scale is not possible.

With the request for the checksums the scale sends a random number, which has to be used from the POS-system for encoding the checksums. This shall prevent a third party from monitoring valid checksums with simple measures.

The random number is a 8-bit-number, the higher nibble (here called Z1) is used for encoding the CS-values, the lower nibble (here called Z2) is used for encoding the KW-values.

The encoding of the CS-values has to be made by rotating them to the left for Z1 bits, the encoding of the KW-values by rotating them to the right for Z2 bits.

- Sample

polynome P: 0x186A3 (fixed)

One sample:

- ECR -> Scale (send record 03 with unit price and tare)

04 02 30 33 1B 30 30 32 39 30 1B 30 30 30 32 03

- Scale -> ECR (send record 10 with random num Z for encoding, Z1=4, Z2=3)

02 31 31 1B 32 34 33 03

- ECR -> Scale (send record 11 with encoding checksum, CS=7114, KW=de66)

04 02 31 30 1B 37 31 31 34 64 65 36 36 03

- Scale -> ECR (send ACK)

06

- ECR -> Scale (send EOT ENQ for scale data)

04 05

- Scale -> ECR (send record 11 with checking ok)

02 31 31 1B 31 03

Calculation:

Source CS: 4711

Encoding CS with Z1: 7114 (rotate CS to left for Z1 bits)

Source KW: f336

Encoding KW with Z2: de66 (rotate KW to right for Z2 bits)

KW = CS / P

Another sample from you: (file name: ABDigiScaleLog.log)

- ECR -> Scale (send EOT ENQ for scale data)

04 05

- Scale -> ECR (send record 10 with random num Z for encoding, Z1=14, Z2=15)

02 31 31 1B 32 45 46 03

- ECR -> Scale (send record 11 with encoding checksum, CS=51c4, KW=e66d)

04 02 31 30 1B 35 31 43 34 45 36 36 44 03

- Scale -> ECR (send ACK)

06

- ECR -> Scale (send EOT ENQ for scale data)

04 05

- Scale -> ECR (send record 11 with checking ok)

02 31 31 1B 31 03

Calculation:

Source CS: 4711

Encoding CS with Z1: 51c4 (rotate CS to left for Z1 bits)

Source KW: f336

Encoding KW with Z2: e66d (rotate KW to right for Z2 bits)

KW = CS / P

SPEC setting:

Please set SPEC7 CS/KW Calculation to 01 (Random Number used for CS/KW).

| 7 | Text length in ASCII (for Checkout-Dialog 02/04 and 06 only) *V2.01 0 - 13 digits 1 - Not specified |  | CS/KW Calculation (for Checkout-Dialog 06 only) *V2.01 00 - Random Number not used for CS/KW 01 - Random Number used for CS/KW 10 - Not used 11 - Not used |
| --- | --- | --- | --- |

Note: Actually the random number will be different each time. If you find the random number is fixed by Z1=14 (45) and Z2=15 (46), the firmware of DS980 isn’t the latest.